| Course Name: | Digital Design Laboratory | Semester: | III |
| --- | --- | --- | --- |
| Date of Performance: |  | Batch No: | A4 |
| Faculty Name: | Kiran Ajetrao | Roll No: | 16010122083 |
| Faculty Sign & Date: |  | Grade/Marks: | /25 |

Experiment No: 2

Title: Binary Adders and Subtractors

Aim and Objective of the Experiment:

To implement half and full adder–subtractor using gates and IC 7483

COs to be achieved:

CO2: Use different minimization technique and solve combinational circuits.

Tools used:

Trainer kits

Theory:

Adder: The addition of two binary digits is the most basic operation performed by the digital computer. There are two types of adder: - Half adder - Full adder Half Adder: Half adder is a combinational logic circuit with two inputs and two outputs. It is the basic building block for the addition of two single-bit numbers. Full adder: A half adder has a provision not to add a carry coming from the lower order bits when multi-bit addition is performed. for this purpose, a third input terminal is added and this circuit is to add A, B, and C where A and B are the nth order bits of the number A and B respectively and C is the carry generated from the addition of (n-1) order bits. This circuit is referred to as full adder. Subtractor: Subtraction of two binary digits is one of the most basic operations performed by digital computer .there are two types of subtractors: - Half subtractor

- Full subtractor

Half subtractor: Logic circuit for the subtraction of B from A where A,B are 1 bit numbers is referred to as half subtract or .the subtract or process has two input and difference and borrow are the two outputs.

Full subtractor: As in the case of the addition using logic gates, a full subtractor is made by combining two half-sub tractors and an additional OR-gate. A full subtractor has the borrow in capability (denoted as BORIN) and so allows cascading which results in the possibility of multi-bit subtraction.

IC 7483

For subtraction of one binary number from another, we do so by adding 2’s complement of the former to the latter number using a full adder circuit.

IC 7483 is a 16 pin, 4-bit full adder. This IC has a provision to add the carry output to transfer and end around carry output using Co and C4 respectively.

2’s complement: 2’s complement of any binary no. can be obtained by adding 1 in 1’scomplement of that no.

e.g. 2’s complement of +(10)10 =1010is

| 1C of 1010 | 0101 |
| --- | --- |
| + | 1 |
| -(10)10 | 0110 |

In 2’s complement subtraction using IC 7483, we are representing negative number in 2’s complement form and then adding it with 1st number.

Implementation Details: Half Adder Block Diagram

Half Adder Circuit

Truth Table for Half Adder

| Inputs |  | Outputs |  |
| --- | --- | --- | --- |
| A | B | A | B |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 |

From the truth table (with steps):

Full Adder Block Diagram

Full Adder Circuit

Truth Table for Full Adder

From the truth table (with steps):

Half Subtractor Block Diagram

Half Subtractor Circuit

Truth Table for Half Subtractor

| A | B | DIFFERENCE(D) | BORROW(Bo) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |

From the truth table (with steps) :

Full Subtractor Block Diagram

Full Subtractor Circuit

Truth Table for Full subtractor

| A | B | BIN | D | BOROUT |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

From the truth table (with steps):

Example:

1) 710 -210 = 510

7	0111

2	0010

1’C of 2

2’C of 2

1101

+ 1

1110

0111 + 1110 1

0101

Pin Diagram IC7483

Adder

Subtractor

Implementation Details

Procedure: - Locate the IC 7483 and 4-not gates block on trainer kit. - Connect 1st input no. to A4-A1 input slot and 2nd (negative) no. to B4-B1 through 4-not gates (1C o no.) - Connect high input to Co so that it will get added with 1C of 2nd no. to get 2C. - Connect 4-bit output to the output indicators. - Switch ON the power supply and monitor the output for various input combinations.

Post Lab Subjective/Objective type Questions:

  - Design a full adder using two half adders. Ans-

  - Perform the following Binary subtraction with the help of appropriate ICs: a. 6-4

b. 5-8

c.   7-9

Ans- (a)

(b)

(c)

Conclusion:

We learnt how to implement half and full adder–subtractor using gates and IC 7483 and understood how to use different minimization technique.

Signature of faculty in-charge with Date: